LSI MegaRAID iMR firmware Flash Process (PRELIMINARY 10/27/10)

Flashing IR mode to iMR mode:

1) Please copy all files to bootable USB disk

2) Run chkir.bat and if the Device ID is 0x72, the is IR mode need update to iMR

3) Run chksas.bat (Write down the SAS address)

4) Run upimrsbr.bat (the Device ID will change to 0x73; it’s iMR mode)

5) Turn off the system and insert the RAID Key: AOC-SAS2-RAID5-KEY

6) Restart the system

7) Run up2imr.bat

8) Reboot the system

9) Run setsas.bat (Use the SAS address from step#2)

10) Reboot the system

11) Goto to WebBIOS and set factory default setting

Flashing iMR mode to IR mode

1) Run chkimr.bat and if the Device Id is 0x73 (It’s in iMR mode)

2) Run chkadd.bat and find SAS address in the adpinfo.txt (This process will take some time)

3) Run upirsbr.bat (the Device ID will change to 0x72; it’s IR mode)

4) Turn off the system and unplug the RAID Key: AOC-SAS2-RAID5-KEY

5) Restart the system

6) Run up2ir.bat

7) Run upsadd.bat with 16 byte SAS address

(Please use the SAS address saved in the adpinfo.txt from step#2)

8) Reboot the system